(SUMO)10-(SIM)5

(SUMO)5-(SIM)10

Grow to OD~0.7 in LB/amp, induce overnight at 18C/175rpm

Purify over Ni column-1)equilibrate and wash in 50mM tris pH 8, 150mM NaCl, 10mM Imidazole, 5mM BME;2)elute in 50mM tris pH 8, 150mM NaCl, 300mM Imidazole, 5mM BME;3)purify over Amylose resin-equilibrate and wash in 50mM tris pH 8, 50mM NaCl, 1mM EDTA, 1mM DTT-elute in 50mM tris pH 8, 50mM NaCl, 1mM EDTA, 1mM DTT, 50mM maltose;4)dilute/desalt to ~50mM NaCl and TeV digest (or not) for 3hrs at room temp;5)purify over Source15Q (5-40% over 30CV);5) purify over SD200 in 10mM tris 8, 50mM NaCl, 1mM EDTA, 1mM DTT