ParM EXPRESSION, PURIFICATION AND LABELLING

The ParM version for MDCC-labelling is ParM His6 I27C K33A T174A T175N C287A

Plasmid

pHis17_ParM _1

Cautionary words: This protocol is provided “as is”, to be used together with any published procedures. It is at the discretion of the user whether to modify or implement as written.

Publications

Kunzelmann, S., and Webb, M. R. (2009) A biosensor for fluorescent determination of ADP with high time resolution, J. Biol. Chem. 284, 33130-33138.

Contact

Martin Webb or Simone Kunzelmann

The Francis Crick Institute

London

mwebb2@mrw.org.uk or simone.kunzelmann@crick.ac.uk

- ParM expression cultures:

Material:

  - BL21 Ai competent cells (Invitrogen)

  - 2 x TY medium

  - Ampicillin (100 g/l stock solution), use at 100 mg/l final concentration

  - Arabinose (200 g/l stock solution, sterile filtered, stable at RT for > 1 year)

  - Resuspension buffer: 30 mM Tris.HCl pH 8.0, 0.1 % (v/v) Triton X-100

ParM expression cultures were always grown from freshly transformed BL21 Ai cells. The description here is for a 1 litre culture:

- Transform BL21 Ai cells (25 µl) with the expression plasmid (0.5 µl of a Miniprep) according to the manufacturers’ protocol

- Inoculate 3 ml 2 x TY medium + 100 mg/l ampicillin with few colonies from plate and grow cells for ~3 hrs at 37°C and 220 rpm.

- Dilute the culture with 20 ml 2 x TY medium + 100 mg/l ampicillin and grow for another ~2 hrs at 37°C and 220 rpm. The culture should now show OD600 between 1 and 1.5.

- Inoculate 2 x 500 ml 2 x TY medium + 100 mg/l ampicillin in 2.5 litres buffled flasks with 10 ml of the starter culture per flask. Grow at 30°C and 150 rpm.

- Make ~20 ml solution of 0.2 g/l arabinose in water and sterile filter.

- When the main culture has reached OD600 between 0.4 and 0.6 (typically after 3 hrs) induce expression by adding 7.5 ml 0.2 g/l arabinose to each flask (final concentration 3 mg/l).

- Grow cells at 30°C and 150 rpm overnight (16 hrs).

- Harvest cells by centrifugation: 3500 rpm, 30 min, 4°C. Weigh cells: typically a 1 litre culture gives about 15 to 20g of cells (wet weight).

- Resuspend cells in resuspension buffer. Use a buffer volume of 2-3 times the cell weight.

- Store cells at -80°C.

- ParM purification:

Material:

  - 5 ml HisTrap HP column (GE Healthcare)

  - Hiprep Superdex 75 column (26/60, 320 ml column volume, GE Healthcare)

  - 20 ml Vivaspin MWCO10 (vivascience)

  - 0.45 µm syringe filter

  - Buffers for Ni-column:

Buffer A: 30 mM Tris.HCl pH 8.0

500 mM NaCl

Buffer B: 30 mM Tris.HCl pH 8.0

25 mM KCl

Buffer B500: 30 mM Tris.HCl pH 8.0

25 mM KCl

500 mM imidazole

pH has to be readjusted after imidazole addition

  - Gelfiltration buffer:

Buffer C: 30 mM Tris.HCl pH 7.5

25 mM KCl

1 mM EDTA

1 mM NaN3

  - PMSF 100 mM in EtOH

  - DTT

Purification was typically done from 10g of cells (wet weight) resulting in a final yield of 200 to 300 mg ParM protein.

- Thaw cell suspension in handwarm waterbath

- Dilute suspension with resuspension buffer (see A) to 50 ml and add 2 mM PMSF

- Break cells by sonication: 4 x 1 min with 1 min break in between, duty cycle 70%, power 6-7

- Clear cell lysate by centrifugation: 45 Ti rotor, 35000rpm, 45 min, 4°C.

- Decant supernatant and supplement supernatant with 500 mM NaCl.

- Filter supernatant through 0.45 µm syringe filter

- Load on HisTrap column equilibrated with buffer A at 3 ml/min. Check pressure and if too high (> 0.3 MPa) reduce flow rate.

- Wash with ~300 ml buffer A at 5ml/min flowrate.

- Wash with ~50 ml buffer B at 5ml/min flowrate.

- Run gradient: 0-250 mM imidazole in buffer B (0-50% buffer B500) over 100ml, collect 5 ml fractions, flow rate 5 ml/min.

- Analyze fraction by SDS-PAGE. Pool fractions containing pure ParM and add 10 mM  DTT.

- Concentrate to a volume < 5ml using Vivaspin20 concentrator.

- Apply concentrated ParM solution to Superdex75 column equilibrated with buffer C. Apply max. 5 ml to the 26/60 column. To remove any precipitate spin for 10 min, 13000rpm, 4°C in an Eppendorf centrifuge before loading.  Run the column at 0.5-2ml/min (depending if overnight or overday) and collect 4 ml fractions.

- The elution profile normally shows 3 peaks: The main peak is the monomeric ParM  at ~210ml. In addition there are small peaks at ~170ml and ~110ml (exclusion volume).

- Analyze the main peak by SDS-PAGE, pool pure fractions and concentrate in Vivaspin to > 1 mM. Determine concentration using 280 = 34380 M-1cm-1.

- Shock-freeze aliquots in liquid nitrogen and store at -80°C.

- Labelling with MDCC:

Material:

  - 5 ml HiTrap Q HP column (GE Healthcare)

  - 20 ml Vivaspin MWCO10 (vivascience)

  - MDCC (20 mM dissolved in DMF)

  - MESNA (freshly prepare a 200 mM stock solution in water)

  - 0.2 µm syringe filter (25 mm HT Tuffryn membrane, Acrodisc)

  - Buffer L  : 30 mM Tris.HCl pH 7.5

25 mM KCl

degassed

  - Buffer L2: 30 mM Tris.HCl pH 7.5

400 mM KCl

  - End-over-end mixer

Labelling of ParM His6 I27C K33A T174A T175A C287A with MDCC was typically done on a scale of 100 mg protein and yields were between 60 and 70%.

- Prepare a solution of 150 µM ParM in buffer L (degassed) and wrap tube in aluminium foil. Choose tube size so that the tube is about ¾ full.

- Add 250 µM MDCC from the stock solution and quickly mix by inverting the tube.

- Put on an end-over-end mixer for 35 min at 22°C.

- Add 2 mM MESNA (from a freshly prepared 200 mM stock solution) and stir for another 10 min.

- Centrifuge the solution to remove precipitate.

- Filter through a 0.2 µm syringe filter.

- Load on HiTrap Q column equilibrated with buffer L. The column can be operated at a flow-rate of 5 ml/min throughout the purification.

- Wash with 100 ml buffer L.

- Run a gradient from 25 mM KCl to 400 mM KCl (0-100% buffer L2) over 200 ml.

- Pool the main peak (should be the first peak) and concentrate labelled protein  in Vivaspin20 to > 1mM.

- Determine the concentration and labelling degree by measuring an absorbance spectrum from 220 to 500 nm. The following extinction coefficients are used to determine the concentration: for the fluorophore MDCC: 430 = 46800 M-1cm-1 and 280 = 7470 M-1 cm-1, for ParM 280 = 34380 M-1cm-1.

- Shock-freeze aliquots in liquid nitrogen and store at -80°C.